Laser micromachining for proton fast ignition laser fusion target fabrication

Nils Schott1,2*, Gabriel Schaumann1,2, Matthias Brönner1,2, Fabian Christ1,2, and Markus Roth1,2

1Institute of Nuclear Physics, TU Darmstadt, Schlossgartenstraße 9, 64289, Darmstadt, Germany

2 Focused Energy GmbH, Im Tiefen See 45, 64293, Darmstadt, Germany

*Corresponding author email: nschott@ikp.tu-darmstadt.de

Laser micromachining with ultrashort laser pulses is an important technique for target production in the realm of laser fusion research. One specific approach of laser fusion is proton fast ignition, which aims to ignite a pre-compressed fuel using a pulse of protons [1]. For this fusion scheme, common target designs use a polymer ablator shell, containing the fusion fuel, and a metallic cone, which is mounted inside the shell pointing towards its center [2]. Since the shell and the cone are made of different materials, one approach to fabricate such a target is to produce both components separately and join them in a second step. In this process, ultrashort pulse laser micromachining can be used to create a hole in the polymer shell for mounting the cone. The application of laser fusion poses strict requirements to the machining quality. Material redeposition on the outside or inside of the shell, as well as material change in the bulk of the shell, must be prevented as they could impair the compression behavior of the capsule [3].

To meet these requirements, we analyze the energy deposition of ultrashort laser pulses in polyvinyl chloride by using two-color pump-probe shadowgraphy. Based on the obtained shadowgraphs and established theory, we identify the dominant ionization and thermalization processes for two different laser pulse intensities. Furthermore, we examine the ablation crater after the interaction with microscope images and show that bulk material change can be mitigated by using large enough intensities to cause significant self-attenuation of the pulse in the sample.

In addition, we analyze material redeposition above and below a trimethylolpropantriacrylat (TMPTA) foil. We conduct studies to reduce material redeposition below the sample by machining on top of a water reservoir. Furthermore, we analyze the potential of burst pulses with MHz and GHz frequencies to reintegrate redeposited particles on the sample surface.

Acknowledgements: We thank Focused Energy GmbH and the EU project IMPULSE (871161) for funding this work.

References: [1] Roth, M., Cowan, T.E., Key, M.H., Hatchett, S.P., Brown, C., et al.: Fast Ignition by Intense Laser-Accelerated Proton Beams. Physical Review Letters 86(3), 436–439 (2001) https://doi.org/10.1103/PhysRevLett.86.436; 
[2] Fernández, J.C., Albright, B.J., Beg, F.N., Foord, M.E., Hegelich, B.M., et al.: Fast ignition with laser-driven proton and ion beams. Nuclear Fusion 54(5), 054006 (2014) https://doi.org/10.1088/0029-5515/ 54/5/054006; 
[3] Craxton, R.S., Anderson, K.S., Boehly, T.R., Goncharov, V.N., Harding, D.R., et al.: Direct-drive inertial confinement fusion: A review. Physics of Plasmas 22(11), 110501 (2015) https://doi.org/10.1063/1.4934714